QingdaoTidaIntemationalTradeco.,Ltd.

This safety datasheet complies with the requirements of Regulation (EC) No.  1907/2006 as amended by Regulation (EC) No 1272/2008 and Commission Regulation (EU) No 453/2010.

Issuing date  01-01-2023             Revision Date  01-01-2023                  Ref  20170101

# SECTION 1: PRODUCT AND COMPANY IDENTIFICATION

Product identifier

Product Code:

SR-235/SR-236/SR-237/SR-2377/SR-240/SR-280/SR-2400/SR-241/SR-9000/SA-50/SA-60/

SA-80/IR-1000/TYR-588/BLR-895/R-996/BLR-698/BLR-886/BLR-631/BLR-852/R-5569/R-5566/R-5568/R-298/R-248/CR-510/CR-210/CR-508/CR-200/CR-211/R-219/R-218/R-216/DHA-100

Product name:    Titanium Dioxide

Substance or Mixture: Mixture

Formula:  TiO2

Identified uses

Recommended use:  White pigment that imparts opacity to coatings, plastics, printing inks andpaper

Uses advised against: For industrial use only

Supplier’s details

Manufacturer: Qingdao Tida International Trade Co., Ltd.

E-mail: amyyue@botianchemcial.com

Address: 13- 101 Hengda Yulan Guoji ,No.702 shanhe Rd.,Chengyang District ,Qingdao ,China

Telephone:Tel : 86-532-55578891    Fax:86-532- 89088579

Emergency telephone number: 86- 13468298184

SECTION 2: HAZARDS IDENTIFICATION

Classification of the substance or mixture

Symbol(s) Not dangerous.

REGULATION (EC) No  1272/2008

Aspiration Toxicity Not classified

Acute Oral Toxicity Not classified

Acute Dermal Toxicity Not classified

Skin Corrosion/Irritation Not classified

Serious Eye Damage/Eye Irritation Not classified

Respiratory Sensitization Not classified

Skin Sensitization Not classified

Germ Cell Mutagenicity Not classified

Carcinogenicity Not classified

Reproductive Toxicity Not classified

Specific target organ systemic toxicity (single exposure) Not classified

Specific target organ systemic toxicity (repeated exposure) Not classified

Acute aquatic toxicity Not classified for acute

Chronic aquatic toxicity Not classified chronic

Other Hazards

Inhalation: Inert nuisance dust.  Temporary drying effect and/or irritation of mucous membranes may  result from excessive exposure. Exposure to dust may aggravate pre-existing respiratory conditions.

Skin: Non-corrosive and non-sensitising. Prolonged contact may result in rashes / irritations due to

-- Page 1 / 7 --

QingdaoTidaIntemationalTradeco.,Ltd.

This safety datasheet complies with the requirements of Regulation (EC) No.  1907/2006 as amended by Regulation (EC) No 1272/2008 and Commission Regulation (EU) No 453/2010.

Issuing date  01-01-2023             Revision Date  01-01-2023                  Ref  20170101

drying of the skin and/or mechanical abrasion related to skin-to-clothing contact or skinto-skin contact. Skin absorption is not applicable.

Eyes: Avoid contact with eyes.

Ingestion: No adverse health effects anticipated by this route during proper industrial handling.

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

| Component | CAS NO. | EC NO: | Content %) |
| --- | --- | --- | --- |
| TITANIUM DIXOIDE | 13463-67-7 | 236-675-5 | 80-99 |
| ALUMINUM HYDROXIDE | 21645-51-2 | 244-492- 7 | <10 |
| AMORPHOUS SILICA | 112926-00-8 | 231-545-4 | <10 |

SECTION 4 FIRST AID MEASURES

Description of necessary first-aid measures

Eye Contact: In the case of contact with eyes, rinse immediately with plenty of water. If symptoms persist, call a physician.

Skin contact: Wash skin with soap and water.  Use of moisturizer maybe helpful.

Ingestion: No adverse health effects anticipated by this route during proper industrial handling. In the event of ingestion, increase intake of liquid in order to flush from body. If symptoms persist, call a

physician.

Inhalation:    Move to fresh air.

Most important symptoms and effects, both acute and delayed

Most Important Symptoms/Effects Nuisance dust.

Indication of immediate medical attention and special treatment needed

Notes to Physician None.

# SECTION 5. FIRE-FIGHTING MEASURES

# Extinguishing media - large fires

# Suitable Extinguishing Media

# Not flammable. Use extinguishing agent suitable for surrounding fire.

Extinguishing media which must not be used for safety reasons

# None.

Special hazards arising from the substance or mixture

# None.

Special Protective Equipment for Firefighters

As in any fire, wear self-contained breathing apparatus and full protective gear.

# SECTION 6. ACCIDENTAL RELEASE MEASURES

Personal precautions, protective equipment and emergency procedures

-- Page 2 / 7 --

QingdaoTidaIntemationalTradeco.,Ltd.

This safety datasheet complies with the requirements of Regulation (EC) No.  1907/2006 as amended by Regulation (EC) No 1272/2008 and Commission Regulation (EU) No 453/2010.

Issuing date  01-01-2023             Revision Date  01-01-2023                  Ref  20170101

Avoid inhalation of dust by arranging adequate ventilation, or use an appropriate dust mask. Avoid excessive contact with the skin.

Other Information

Refer to Section  13 for disposal considerations.

Environmental precautions

Prevent further leakage or spillage if safe to do so.  Use dyking or absorbant to prevent run-off from entering waterways.

Methods and materials for containment and cleaning up

Methods for Cleaning up

Use any feasible mechanical means (e.g. vacuuming, absorbent material).

# SECTION 7. HANDLING AND STORAGE

Precautions for Safe Handling

Handling

Take precautionary measures against static discharges. Minimize inhalation of dust and contact with

## the skin.

Conditions for safe storage, including any incompatibilities

To guarantee the quality and properties of the product keep protected from humidity and bad weather conditions. Avoid prolonged

storage. Product maybe packaged in normal commercial packaging; paper or plastic material. Caution: The stacking height must

not exceed 3 pallets. Do not store flexible containers on top of each other. Refer to the detailed list of incompatible materials

(section 10  "Stability/Reactivity").

Specific End Uses

Specific Uses Take precautions against the discharge of static electricity during powder handling operations.

Exposure Scenario None required as mixture is not hazardous.

Other Guidelines None.

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Components with workplace control parameters

| CHEMICAL NAME | EU | THE UNITED KINDOM | FRANCE | SPAIN | GERMANY |
| --- | --- | --- | --- | --- | --- |
| TITANIUM DIXOIDE |  | STEL: 30 mg/m3 STEL: 12 mg/m3 TWA: 10 mg/m3 TWA: 4 mg/m3 | VME: 10 mg/m3 | VLA-ED: 10 mg/m3 |  |

| CHEMICAL NAME | ITALY | PORTUGAL | THE NETHERLANDS | FINLAND | DENMARK |
| --- | --- | --- | --- | --- | --- |
| TITANIUM DIXOIDE |  | TWA: 10 mg/m3 | MAC: 10 mg/m3 MAC |  | TWA: 6 mg/m3 |

-- Page 3 / 7 --

QingdaoTidaIntemationalTradeco.,Ltd.

This safety datasheet complies with the requirements of Regulation (EC) No.  1907/2006 as amended by Regulation (EC) No 1272/2008 and Commission Regulation (EU) No 453/2010.

Issuing date  01-01-2023             Revision Date  01-01-2023                  Ref  20170101

| CHEMICAL NAME | AUSTRIA | SWITZERLAND | POLAND | NORWAY | IRELAND |
| --- | --- | --- | --- | --- | --- |
| TITANIUM DIXOIDE | STEL 10 mg/m3 MAK: 5 mg/m3 | MAK: 3 mg/m3 | NDS: 10.0 mg/m3 | TWA: 5 mg/m3 STEL: 10 mg/m3 | TWA: 10 ) mg/m3(respirable fraction |

| CHEMICAL NAME | BELGIUM | CZECH REPUBLIC | GREECE | SWEDEN | UNITED STATES |
| --- | --- | --- | --- | --- | --- |
| TITANIUM DIXOIDE | TWA: 10 mg/m3 |  | TWA: 10 mg/m3 TWA: 5 mg/m3 | 5 mg/m3 (total dust) | TLV-TWA: 10 mg/m3 TWA: 15 mg/m3 |

Derived No Effect Level (DNEL):  10 mg/m3

Predicted No Effect Concentration (PNEC):   Marine water: >1 mg/l; sediment: >100 mg/kg Freshwater: = 0. 127 mg/l; sediment: >1000 mg/kg

Exposure controls

Engineering Measures: Good natural ventilation will be sufficient in most circumstances. Local

exhaust ventilation maybe necessary if airborne dust concentration approaches the exposure limit.

Personal protective equipment

Eye Protection: Safety glasses with side-shields or goggles.

Hand Protection: Impervious gloves.

Skin and Body Protection: Long sleeved clothing.

Respiratory Protection: In the event of insufficient ventilation: Respiratory protective device with a particles filter.

Environmental Exposure Controls:   Do not allow material to contaminate ground water system.

## SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES

Physical State:    Solid

Appearance:    White

Odor:    None

pH : Not applicable

Vapor Pressure:    Not applicable

Vapor Density:    Not applicable

Viscosity : Not applicable

Boiling Point/Range:    Not applicable

Melting Point/Range:    1830 ° C

Decomposition  Temperature:    No data available

Explosive Properties:    Not applicable

Flash Point : Not applicable

Oxidizing Properties:    Not applicable

Evaporation Rate:    Not applicable

Flammability (solid, gas):    Not applicable

Water Solubility:    Insoluble in water

Solubility in other solvents:    Insoluble in ether

Specific Gravity:   4.0 - 4.2

VOC Content(%):    None

Partition coefficient: n-octanol/water No data available

Autoignition  Temperature:    Not applicable

-- Page 4 / 7 --

QingdaoTidaIntemationalTradeco.,Ltd.

This safety datasheet complies with the requirements of Regulation (EC) No.  1907/2006 as amended by Regulation (EC) No 1272/2008 and Commission Regulation (EU) No 453/2010.

Issuing date  01-01-2023             Revision Date  01-01-2023                  Ref: 20170101

## SECTION 10. STABILITY AND REACTIVITY

Reactivity

## None.

Chemical Stability

Stable under normal conditions.

Possibility of Hazardous Reactions

None under normal processing.

Conditions to Avoid

## None.

Materials to Avoid

## None.

Hazardous Decomposition Products

## None.

Shelf-life

Two years in normal conditions from production date.

## SECTION 11. TOXICOLOGICAL INFORMATION

Acute toxicity

Product Information  These products do not present an acute toxicity hazard based on known or supplied information.

Irritation   Same as a nuisance  dust.

Ingestion  No known toxic effect.

Skin  Contact     No known effect.

Inhalation  No known toxic effect.

Eye Contact     No known toxic effect.

Corrosivity   Individuals with sensitive skin may experience skin drying on prolonged or repeated

exposure.

Component Information

Chronic Effects

Carcinogenicity  Titanium Dioxide is listed by IARC as possibly carcinogenic to humans (Group 2B). This listing is based on inadequate evidence of carcinogenicityin humans and sufficient evidence in  experimental animals.

Sensitization: No evidence of skin or respiratory sensitisation.

Mutagenic Effects:    None known.

Reproductive Toxicity:    None known.

Developmental Toxicity   None known.

STOT - single exposure  : Not classified.

STOT - repeated exposure   Not classified.

Target Organ Effects   In lifetime inhalation studies of rats, airborne respirable-size titanium dioxide particles have been shown to cause lung tumors at concentrations associated with substantial

particle lung burdens and consequential pulmonary overload and inflammation. However, other

laboratory animals such as mice and hamsters did not develop lung tumors under similar testing with titanium dioxide. Furthermore, human epidemiology studies do not suggest an association between

occupational exposure to titanium dioxide and risk for cancer.

-- Page 5 / 7 --

QingdaoTidaIntemationalTradeco.,Ltd.

This safety datasheet complies with the requirements of Regulation (EC) No.  1907/2006 as amended by Regulation (EC) No 1272/2008 and Commission Regulation (EU) No 453/2010.

Issuing date  01-01-2023             Revision Date  01-01-2023                  Ref: 20170101

Aspiration Hazard   None known.

Endocrine Disruptor Information    These products do not contain any known or suspected endocrine

disruptors.

## SECTION 12. ECOLOGICAL INFORMATION

Toxicity

Ecotoxicity effects

## None known.

Persistence and degradability

Titanium dioxide is persistent and does not biodegrade.

Bioaccumulative potential

Does not bioaccumulate.

Mobility in soil

There is no evidence of mobility of these products (solid particles).

PBT and vPvB assessment

Not PBT and not vPvB.

## SECTION 13. DISPOSAL CONSIDERATIONS

Waste treatment

Waste from Residues / Unused Products

These products are not considered hazardous for disposal into sanitary landfill or industrial waste disposal landfill. Please review appropriate national and local waste regulations.

Contaminated Packaging

Contaminated packages are not considered hazardous for disposal into sanitary landfill or industrial

waste disposal landfill. Please review appropriate national and local waste

regulations.

EWC Waste Disposal No EWC Code: 06  11 99. Not classified as a hazardous waste.

## SECTION 14. TRANSPORT INFORMATION

IMDG/IMO   Not regulated

RID Not regulated

ADR  Not regulated

ICAO   Not regulated

IATA   Not regulated

## SECTION 15. REGULATORY INFORMATION

These products are not classified as dangerous according to Directives 67/548/EEC and 1999/45/EC.

Labeling

Symbol(s) Not dangerous.

-- Page 6 / 7 --

QingdaoTidaIntemationalTradeco.,Ltd.

This safety datasheet complies with the requirements of Regulation (EC) No.  1907/2006 as amended by Regulation (EC) No 1272/2008 and Commission Regulation (EU) No 453/2010.

Issuing date  01-01-2023             Revision Date  01-01-2023                  Ref: 20170101

International Inventories

USA (TSCA) Complies                                                                                                                                .

The products on this MSDS comply with the country chemical control inventories listed below

European Union (EINECS) Complies

Canada (DSL/NDSL)  Complies

Philippines (PICCS)  Complies

Japan (ENCS)  Complies

China (IECSC) Complies

Australia (AICS)  Complies

Korea (KECL) Complies

New Zealand (NZIoC) Complies

Legend

TSCA - United States  Toxic Substances Control Act Section 8(b) Inventory

EINECS/ELINCS - European Inventory of Existing Commercial Chemical Substances/EU List of Notified Chemical Substances

DSL/NDSL - Canadian Domestic Substances List/Non-Domestic Substances List

PICCS - Philippines Inventory of Chemicals and Chemical Substances

ENCS - Japan Existing and New Chemical Substances

IECSC - China Inventory of Existing Chemical Substances

AICS - Australian Inventory of Chemical Substances

KECL - Korean Existing and Evaluated Chemical Substances

NZIoC - New Zealand Inventory of Chemicals

Chemical Safety Assessment Not hazardous.

## SECTION 16. OTHER INFORMATION

Text of R-phrases mentioned in Section 3

Not applicable.

Sources of key data used to compile the Material Safety Data Sheet

REACH Registration dossier and chemical safety report

www.ChemADVISOR.com/

Revision Note

Reason for Revision Publish MSDS in accordance  with  Commission Regulation (EU) No 453/2010 (REACH) and Regulation (EC) No  1272/2008 (CLP).

This safety datasheet complies with the requirements of Regulation (EC) No.  1907/2006 as amended by Regulation (EC) No 1272/2008 and Commission Regulation (EU) No 453/2010.

Disclaimer

The information provided on this MSDS is correct to the best of our knowledge, information and belief at the date of its publication.  The information given is designed only as a guide for safe handling, use,  processing, storage, transportation, disposal and release and is not to be considered as a warranty or quality specification.  The information relates only to the specific material designated and may not be

valid for such material used in combination with any other material or in any process, unless specified in the text.

-- Page 6 / 7 --